METHOD & IMPLEMENTATION · FIRE PERIMETER EVOLUTION · V2

Perimeter evolution under observed rate-of-spread vectors

A single method for filling the gap between two observed fire perimeters, driven by the measured rate-of-spread vector rather than by a modelled fuel map. The two prototype algorithms turn out to be special cases of it — this document shows why, explains the mathematics it rests on, and specifies how to build it.

Input   a pair of observed perimeters; a bulk ROS vector per perimeter where available (§7 if not)

Output  a smooth time-of-arrival grid over the region between them, for WRF-SFIRE (TIGN_G)

Solver  closed form — no PDE integration, no iteration, no tuning parameters

Status  core results verified numerically against analytic test cases

Revision note.  v2 corrects the wavelet parameterisation in §2.5 (the ignition point sits at the ellipse’s rear focus, so a, b and c are not independent), resolves the wind-speed unit question against Finney (1998), reframes the C6 discussion in §5, adds §7 on perimeters that carry no ROS vector, qualifies the synthetic anisotropy result in §8, and adds a reference list.

# 1.  The problem and the constraints it imposes

Fire mapping is flown for suppression, not for research. Perimeters arrive at irregular, operationally convenient times; satellite platforms lack the spatial and temporal resolution to fill the gaps, and aircraft face access restrictions during active burning. The task is to reconstruct what the fire did between two observations, well enough that the result can constrain a coupled atmospheric simulation.

That framing imposes seven constraints, and they are what the rest of this document is accountable to.

|  | Constraint | What it rules out |
| --- | --- | --- |
| C1 | Sparse input | Only intermittent observed perimeters are available. The method cannot assume a dense time series, nor calibrate against intermediate truth it does not have. |
| C2 | Observations are hard constraints | The evolution must begin exactly on the observed initial perimeter and end exactly on the observed final one. Approximate convergence is a failure, not a tolerance. |
| C3 | Continuous, smooth output | The product is a continuous time-of-arrival field, not a set of frames. Discontinuities in it become discontinuities in simulated heat release. |
| C4 | Must not contaminate the atmospheric signal | The grid exists so that WRF-SFIRE runs can isolate atmospheric effects. Any fuel or terrain physics baked into the interpolation is a signal that can no longer be attributed to the atmosphere. |
| C5 | Spotting | Components of the final perimeter detached from the initial one must be reached. A method that requires a continuous path fails here. |
| C6 | Physically plausible spread | Speed and direction must vary across the front. Uniform or instantaneous advance is not acceptable even if the endpoints match. |
| C7 | Monotone over a defined period | Burned area only grows, over a user-defined interval, normalised so frames can be requested at any progress fraction. |

## Where this sits relative to published work

The same problem has been addressed in the same model. Kochanski et al. (2023) compare three ways of initialising WRF-SFIRE from observed perimeters and find that a gradual spin-up driven by an interpolated fire arrival time clearly outperforms instantaneous ignition — reducing day-one fire growth error on the Creek Fire from 273% to 94%, and avoiding the unphysical updrafts (over 52 m/s) that instantaneous ignition produced. That result is independent support for C3 and C4.

Their arrival-time field, however, comes from a bi-harmonic spline interpolation of the 2D arrival data: a generic smoothness prior with no endpoint guarantee and no fire physics in it. That is the baseline this method is measured against, and it is the reason the two properties developed here — structural endpoint exactness and a metric derived from observation — are the contribution rather than the machinery around them. Farguell et al. (2021) reach a comparable product from a different direction, fitting a support-vector boundary to satellite active-fire detections; they explicitly do not interpolate between perimeters, and use IR perimeters only for validation.

C4 deserves emphasis because it drives a design decision that would otherwise look like a simplification. The purpose of the arrival grid is to hold the fire fixed so that atmospheric behaviour can be studied against it. If the interpolation embeds a fuel-and-terrain spread model, then part of what the simulation produces is that model talking to itself. The interpolation should assume as little as possible and use observations wherever they exist — which is exactly what a measured ROS vector allows.

# 2.  Concepts and notation

This section explains the machinery. None of it is exotic, but the vocabulary is scattered across three literatures — level-set methods, fire behaviour modelling, and Finsler geometry — and the same object often has a different name in each.

## 2.1  Representing a perimeter: level sets

A perimeter could be stored as a list of vertices, but then every operation becomes bookkeeping about which vertex connects to which — and a perimeter that splits, merges, or gains a detached spot fire breaks that bookkeeping. The level-set representation avoids it entirely.

Define a field φ(x) over the whole grid whose value at each cell is the distance from that cell to the perimeter, signed negative inside and positive outside. This is the signed distance function, or SDF. The perimeter is then implicit — it is the set of points where φ = 0, the zero-level contour. Curves stored this way merge, split, and appear from nothing with no special handling, which is precisely what makes C5 tractable.

## 2.2  Arrival time: one field instead of many frames

The second idea is to stop thinking in frames. Rather than a curve moving through time, compute a single field T(x) giving the time at which the fire reaches each cell. Every frame of the animation is then just a contour of that one field.

This is not only a convenience — it is the deliverable. TIGN_G, the ignition-time array WRF-SFIRE consumes, is a time-of-arrival grid. So the entire method reduces to one question: how do we compute T well?

## 2.3  Metrics: what "travel time" means

A metric is a rule assigning a travel time to every displacement. Three kinds matter here:

- Euclidean — time = length / speed, the same in every direction and at every location. This is what "normalised front speed" amounts to.

- Isotropic but spatially varying — the speed depends on where you are, but not on which way you are facing. A Rothermel rate-of-spread map used as a scalar speed field is this kind.

- Anisotropic — the speed depends on direction as well. This is the kind fire under wind actually needs, and it is the reason a scalar speed map cannot represent a ROS vector: a single number per cell has nowhere to put a bearing.

## 2.4  Huygens’ principle and the wavelet

Treat every point on the burning front as itself a new ignition. After a short time each has grown into a small shape; the new front is the outer envelope of all of them. This is Huygens’ principle, borrowed from optics, and it is how fire spread models are normally constructed.

The consequence is powerful: the entire spread model is encoded in one shape — what a single ignition becomes in unit time. Call it the wavelet. Choose the wavelet and you have chosen the metric; everything else follows.

Two cautions about the word. It has nothing to do with wavelet transforms — no Haar, no Daubechies, no multiresolution analysis; the term is borrowed from Huygens’ optics, where each point of a wavefront emits a secondary wavelet. Given the audience, the fire-literature alternatives elliptical template or fire shape may be safer in the final write-up. And the wavelet is a local law, not a prediction of a fire’s shape: a real perimeter is the envelope of thousands of these, and is not itself an ellipse.

## 2.5  The fire ellipse

Empirically, a point ignition under steady wind grows into an ellipse — and the ignition point is not at the ellipse’s centre. It sits back toward the upwind end. That offset is exactly what makes heading spread faster than backing.

Left: the wavelet and its three parameters. Right: the same shape answering two different questions.

Three quantities describe it, all in metres per second because the wavelet is a unit-time shape:

head = c + a  ,   backing = a − c  ,   flanking = b

a is the semi-major axis, b the semi-minor, c the offset of the ignition point behind the ellipse centre.

They are not independent. The ignition point sits at the ellipse’s rear focus, which fixes the offset at the focal distance:

c = √( a² − b² )    ,    c / a = eccentricity

So the wavelet has two free parameters, not three — and c < a holds automatically for any finite LB.

This is also where the standard head-to-back relation comes from. Substituting the focal distance into (a + c)/(a − c) and writing LB = a/b gives Finney’s Eq. 14 directly, so the formula below is a consequence of the focus assumption rather than an independent empirical fit.

You observe one vector, which gives the heading û and the head rate |v|. One further number — the length-to-breadth ratio LB — closes the system. It is the ellipse’s elongation, conventionally taken from midflame wind speed via the Anderson (1983) relation as implemented in Finney (1998), Eq. 13. From LB comes the head-to-back ratio HB, and from there everything else:

HB = ( LB + √(LB² − 1) ) / ( LB − √(LB² − 1) )

Then R_back = |v| / HB, a = (|v| + R_back)/2, c = (|v| − R_back)/2, and b = a / LB. This parameterisation is self-consistent with the focus condition above: b = a/LB and b = √(a² − c²) agree exactly, verified across LB from 1.2 to 10.

On units — Finney (1998), Eq. 13 gives LB = 0.936·e^(0.2566U) + 0.461·e^(−0.1548U) − 0.397 with U as midflame windspeed in m s⁻¹. The trailing −0.397 is Finney’s modification of Anderson’s original relation, not part of it, so take the equation in this form rather than reconstructing it from the 1983 paper.

One consequence worth stating so it is not mis-sold: because c/a is the eccentricity, the condition c < a is satisfied for every finite LB. It is not a physical constraint on wind speed, and no wind violates it. Keep the check in code as a guard against a corrupted parameterisation, but do not present it as a limit of applicability. What it does mark is the degenerate limit: as LB → ∞ the backing rate goes to zero.

## 2.6  Two views of the wavelet: support function and gauge

The same ellipse answers two different questions, and which one you ask determines what kind of algorithm you end up writing.

The support function asks: the front here is facing direction n̂ — how fast does it advance? It is the perpendicular distance from the ignition point to the wavelet’s tangent line in that direction:

F(ψ) = c·cos ψ + √( a²cos²ψ + b²sin²ψ )

ψ is the angle between the front normal and û. This is the form the level-set and Eikonal literature uses, because a PDE needs a normal speed.

The gauge — also called the Minkowski functional — asks the inverse question: here is a displacement v, how long until the fire covers it? It is the smallest scaling of the wavelet that swallows v:

τ(v) = min { t > 0 : v / t ∈ E }

The two are convex duals of one another and describe the same ellipse. We need the gauge, because arrival time from a source is a minimum of τ over the source — which is a direct computation rather than a differential equation. That single choice is what removes the PDE solver from this method.

## 2.7  Riemannian, Randers, Finsler

Vocabulary worth having, because it is how to cite this work and how to find prior art.

- Riemannian — the wavelet is an ellipse centred on the ignition point. Direction matters, but symmetrically: a path costs the same travelled either way.

- Randers — the ellipse is offset. This is a Riemannian metric plus a uniform drift, and it breaks the symmetry: downwind is cheaper than upwind. Wind-driven fire spread is the standard physical example.

- Finsler — the general class, any metric whose unit ball varies with direction. Riemannian and Randers are both special cases.

So the object at the centre of this method is a Randers metric, and that is the term to use in the write-up.

## 2.8  The Eikonal equation, and why we do not solve it

The arrival-time field obeys

|∇T| = 1 / F

The more steeply T rises, the slower the front is moving there.

This is the Eikonal equation, normally solved numerically by fast marching. Two things are worth knowing about it here.

First, fast marching is only valid for an isotropic F. Once the speed depends on direction the standard algorithm no longer applies, and the obvious workarounds are worse than they look — §9 records what was tested and rejected.

Second, and more usefully: when the metric is the same everywhere, the Eikonal equation has an exact closed-form solution, and that solution is the gauge. Dropping the modelled fuel map (C4) is what makes the metric homogeneous, and homogeneity is what turns a numerical PDE problem into arithmetic.

## 2.9  Glossary

| Term | Meaning here |
| --- | --- |
| Signed distance function (SDF) | A grid field holding distance to the perimeter, negative inside, positive outside. |
| Zero-level contour | The perimeter itself: where the SDF equals zero. |
| Arrival time / time of arrival | Field giving the moment the fire reaches each cell. The deliverable. |
| Metric | Rule assigning a travel time to a displacement. |
| Isotropic / anisotropic | Independent of direction / dependent on direction. |
| Wavelet | The shape a single point ignition grows into in unit time. Encodes the whole spread model. |
| Support function F(ψ) | Normal speed of the front when it faces at angle ψ to the heading. |
| Gauge τ(v) | Time for the fire to cover displacement v. Dual to the support function. |
| Randers metric | Riemannian metric plus a uniform drift; asymmetric, so downwind is cheaper. |
| LB / HB | Length-to-breadth and head-to-back ratios of the fire ellipse. |
| Medial axis | The skeleton of a shape — points equidistant from two or more boundary points. Any quantity computed by "nearest boundary point" is discontinuous across it. |
| Monotone / nested | Burned area only grows; each frame contains the previous one. |
| Progress field s(x) | Normalised fire arrival time: 0 on the initial perimeter, 1 on the final one. "Blend" is informal shorthand; prefer "bidirectional arrival-time normalisation" for the operation and "normalised fire arrival time" for the field. |

# 3.  How the two prototypes relate: two knobs

The poster presents two algorithms with a trade-off: the linear method converges on the final perimeter exactly but spreads unrealistically; the physics-guided method spreads realistically but does not converge exactly. That trade-off is not intrinsic. The two prototypes differ in two independent ways at once, and each got one of them right.

## The linear method is a distance blend in disguise

Write its zero level set explicitly. It sits where

(1 − s)·φ_i + s·φ_f = 0    ⟹    s = φ_i / ( φ_i − φ_f )

In the region between the perimeters φ_i is positive and φ_f is negative, so φ_i is the distance to the initial perimeter and −φ_f the distance to the final one. Substituting:

s(x) = d_i(x) / [ d_i(x) + d_f(x) ]

Verified numerically on irregular perimeters: agreement to 0.0, machine precision.

So the linear method already computes a two-sided blend. Its only limitation is that the distances are Euclidean — every direction equally fast, everywhere. Replace those with travel times under any other metric and the same expression becomes the general method.

## The two knobs

Every version of this method is that one expression with two independent settings.

- Knob 1 — the metric. What "distance" means: Euclidean, isotropic modelled speed, or the ellipse from the observed ROS vector. This controls whether the intermediate spread looks physical (C6).

- Knob 2 — the clock. How raw travel time becomes a 0-to-1 progress value. Two-sided measures both the journey travelled and the journey remaining; one-sided measures only the outbound leg and divides by an estimate of the total, obtained by looking up the arrival time at the nearest point of the final boundary. This controls endpoint exactness (C2).

|  | Euclidean metric | Modelled isotropic | Ellipse from ROS |
| --- | --- | --- | --- |
| One-sided clock | — | the physics-guided prototype | — |
| Two-sided clock | the linear prototype | never built | this method |

Read across the table and the poster’s trade-off dissolves. Endpoint exactness came from the two-sided clock, not from linearity. Realistic spread came from the metric, not from Rothermel specifically. Because the two prototypes happened to differ in both knobs at once, the two properties looked welded together — but they are independent, and setting both well is not a compromise between the prototypes. It is the cell of the table nobody had filled.

This also supplies a free regression test: run the new code with a Euclidean metric and it must reproduce the linear prototype. Measured agreement is 1.25e−2 maximum, 5.2e−3 median — the vertex-discretisation floor.

## Why the one-sided clock fails

Worth understanding rather than just avoiding. The one-sided normalisation divides arrival time by a total estimated as "the arrival time at the nearest point of the final boundary". That lookup is constant across whole regions and jumps wherever the nearest boundary point changes — that is, across the medial axis of the final perimeter. The ratio inherits those jumps, which violates C3 directly, and the level set where the ratio equals one is not the observed perimeter, which violates C2. Measured overshoot on a test case: s reaches 1.083 inside the final perimeter, so the "final" contour lands well inside it.

# 4.  The method

Four stages.

### Stage A — Grid and signed distance functions

Rasterise both observed perimeters onto one grid in metres, in a locally equidistant projection, and convert each to a true SDF.

### Stage B — Build the wavelet from the observed vector

Take û and |v| from the ROS vector, close the ellipse with LB, and form (a, b, c) as in §2.5. This is the whole of the physics input.

### Stage C — Two arrival-time solves

Outward from the initial perimeter under the metric, and inward from the final perimeter under the reversed metric — the same ellipse with the offset negated, because receding traverses the same paths backwards. Each is a direct gauge minimisation, not an integration.

### Stage D — Blend and normalise

s(x) = T_f(x) / [ T_f(x) + T_b(x) ]

Left: T_f is the journey already travelled to reach x, T_b the journey remaining from x to the final perimeter. Right: the resulting progress field, with the 0.25, 0.50 and 0.75 contours drawn.

T_f + T_b is the total travel time of the path through x, so s is the fraction of that journey completed when the fire arrives. The endpoints follow structurally: on the initial perimeter T_f = 0 so s = 0; on the final perimeter T_b = 0 so s = 1. Neither is fitted, and both hold under any metric — which is why changing the metric costs nothing in endpoint accuracy.

The denominator is computed per point, not globally: every cell has its own total journey time, so a cell on a fast downwind path and one on a slow backing path can both read s = 0.5 despite very different absolute times. That is the correct behaviour for interpolating between two fixed observations, and it is what the nearest-boundary lookup was trying to approximate.

Converting to seconds for output is then a single multiplication by the observed interval.

# 5.  Constraint check

|  | How the method satisfies it |
| --- | --- |
| C1 | Uses only the two observed perimeters and their vectors. No intermediate truth is required, and none is assumed. |
| C2 | Endpoint exactness is structural, not fitted — T_f = 0 and T_b = 0 force s = 0 and s = 1. Measured residual 0.022 on both contours, which is one grid cell. |
| C3 | s is continuous everywhere between the perimeters. There is no nearest-neighbour lookup, so nothing jumps across the medial axis. |
| C4 | The only physics entering is the measured ROS vector plus one shape parameter. No fuel model, no terrain, nothing for the simulation to talk to itself about. |
| C5 | A gauge distance is defined between any two points regardless of connectivity, so detached components receive finite arrival times automatically — the same reason the linear prototype handled spotting. |
| C6 | The ellipse gives direction-dependent speed: heading, flanking and backing all differ, and the intermediate fronts are biased downwind rather than advancing uniformly. |
| C7 | s is dimensionless progress on [0, 1], so any frame can be requested; monotonicity is verified and additionally enforced by accumulation. |

## On the absence of fuel and terrain

The method gives direction-dependent spread but not spatial variation — the metric is the same everywhere, so the front does not slow in heavy fuel or divert around a break. This is a design decision, not a shortfall, and the argument for it is worth making explicitly rather than conceding.

The observed perimeters already carry the integrated effect of fuel and terrain. If a break stopped the fire, the final perimeter shows it, and the method reproduces that exactly because it is a hard constraint (C2). A fuel model added on top is therefore redundant with the observation, and can actively disagree with it — pushing intermediate fronts somewhere the endpoints contradict.

For this application the argument is sharper still. WRF-SFIRE computes Rothermel spread itself once the spin-up ends. Embedding Rothermel in TIGN_G applies the same fuel physics twice — once as the boundary condition and once inside the model — after which no plume feature can be attributed cleanly to the atmosphere. That is C4 stated concretely.

What remains is a scope statement rather than an apology: the perimeters constrain the extent, the ROS vector constrains the direction, and what is left unresolved is sub-interval variation in spread rate from heterogeneity — bounded by the observation interval, and shrinking as perimeters become more frequent. §9 covers the one case, non-burnable barriers, where revisiting this is justified.

# 6.  Implementation

The prototypes established the representation and the workflow; the grid, solver and module structure below are new. Steps are ordered so each leaves the code runnable.

## Step 1 — Grid and projection

One GridSpec — CRS, bounds, cell size — shared by everything downstream. The projection now carries physical meaning: the ROS vector is in metres per second and it is the only physical input, so the grid must be locally equidistant. Web Mercator is conformal but not equidistant; at the Mill Fire’s latitude its scale factor is about 1.33, which would inflate every distance by a third and corrupt the one quantity the method depends on. Use a local UTM zone (10N for northern California) or an equidistant azimuthal projection centred on the fire.

def sdf(mask, dx):

"""Signed distance in metres: negative inside, positive outside."""

return (distance_transform_edt(~mask, sampling=dx)

- distance_transform_edt(mask,  sampling=dx))

## Step 2 — Wavelet parameters from the vector

def ellipse_params(ros, LB):

"""(u, v) m/s plus length-to-breadth -> heading and wavelet axes."""

w  = np.asarray(ros, float)

Rh = np.linalg.norm(w)                  # observed head rate

if Rh < 1e-9:  raise ValueError("zero ROS vector")

if LB < 1.0:   raise ValueError("LB must be >= 1")

u  = w / Rh                             # unit heading

HB = (LB + np.sqrt(LB**2 - 1)) / (LB - np.sqrt(LB**2 - 1))

Rb = Rh / HB                            # backing rate

a  = (Rh + Rb) / 2.0                    # semi-major

c  = (Rh - Rb) / 2.0                    # offset along u

b  = a / LB                             # semi-minor

assert c < a          # guard only: always true for finite LB

return u, a, b, c

For a pair, use the mean of the two perimeters’ vectors; §9 covers the ramp between them. Where a perimeter carries no vector at all, §7 gives the alternatives.

## Step 3 — The gauge solver

Derivation first, because the implementation is otherwise opaque. The condition that x is reached from y by time t is that (x − y) lies inside the wavelet scaled by t:

| A⁻¹ ( v/t − c·û ) | ≤ 1 ,    v = x − y

Rotate v into the û frame and stretch by A⁻¹ to get p; write q = (c/a, 0). The condition becomes |p/t − q| ≤ 1, which is a quadratic in t, so the smallest admissible t is explicit:

τ(v) = [ −p·q + √( (p·q)² + (1 − |q|²)·|p|² ) ] / ( 1 − |q|² )

Requires |q| < 1, which is the same as c < a. Since c/a is the eccentricity, this holds for every finite LB.

Minimising over the source boundary rather than the whole source region is exact: τ scales linearly, so for any point outside the source the closest point in the metric always lies on the boundary. The direction checks fall straight out — along û the expression reduces to d/(a + c), against û to d/(a − c), and perpendicular to d divided by b√(1 − c²/a²).

def boundary_vertices(mask, X, Y):

bnd = mask & ~binary_erosion(mask)

iy, ix = np.nonzero(bnd)

return X[iy, ix], Y[iy, ix]

def gauge_arrival(sx, sy, uhat, a, b, c, X, Y, chunk=4000):

"""Exact arrival time under a homogeneous elliptical metric."""

ux, uy = uhat

qx  = c / a

den = 1.0 - qx*qx

if den <= 0:

raise ValueError("c >= a: backing rate is non-positive")

gx, gy = X.ravel(), Y.ravel()

T = np.empty(gx.size)

for i0 in range(0, gx.size, chunk):        # chunked: the full outer

i1 = min(i0 + chunk, gx.size)          # product will not fit

vx = gx[i0:i1, None] - sx[None, :]

vy = gy[i0:i1, None] - sy[None, :]

px = ( vx*ux + vy*uy) / a              # rotate into the u frame,

py = (-vx*uy + vy*ux) / b              #   then stretch by A^-1

pq = px * qx

T[i0:i1] = ((-pq + np.sqrt(pq*pq + den*(px*px + py*py)))

/ den).min(axis=1)

return T.reshape(X.shape)

Cost is cells × boundary vertices: 0.23 s, 1.0 s and 3.5 s per solve at 201², 401² and 801² with 292 source vertices. Keep the chunk size modest — the intermediate array is chunk × vertices, and an unchunked 501² grid against 1200 vertices will exhaust memory.

## Step 4 — The two solves and the blend

vi = boundary_vertices(mask_start, X, Y)

vf = boundary_vertices(mask_end,   X, Y)

u, a, b, c = ellipse_params(0.5*(np.array(ros_i) + np.array(ros_f)), LB)

T_f = gauge_arrival(*vi,  u, a, b, c, X, Y)   # outward, forward metric

T_b = gauge_arrival(*vf, -u, a, b, c, X, Y)   # inward,  offset negated

s = T_f / np.maximum(T_f + T_b, 1e-12)

s[mask_start] = 0.0

s[~mask_end]  = np.nan

Negating û for the backward solve reverses the offset while leaving the ellipse shape unchanged, which is what "reversed metric" means concretely. Getting this wrong does not throw — it produces a plausible-looking field with the wrong intermediate ordering, so cover it with the monotonicity test in §7.

## Step 5 — Frames and monotonicity

burned = np.zeros_like(s, bool);  frames = [];  violations = 0

for lv in np.linspace(0, 1, num_steps + 1):

m = (s <= lv) & mask_end

if (burned & ~m).any(): violations += 1

burned |= m                          # running union enforces C7

frames.append(burned.copy())

Report the violation count rather than silently absorbing it. It was zero in every test, so a nonzero value is a signal that the two metrics disagree, not a routine correction.

## Step 6 — Export

t_ign(x) = t_i + s(x) · ( t_f − t_i )

Seconds from simulation start, on the fire grid, with a large sentinel outside the final perimeter for cells that never ignite. Keep every in-perimeter value strictly greater than one model time step, or SFIRE rejects the array during initialisation.

## Step 7 — Chain the pairs

For a full time series, run each consecutive pair, map each s into absolute seconds, and take the elementwise minimum across segments. Verify that each segment’s final mask matches the next segment’s initial mask before chaining — a mismatch means the observations disagree with each other, and smoothing over it hides a data problem rather than solving one.

# 7.  When the perimeters carry no ROS vector

Because the metric is a parameter rather than a fixed part of the algorithm (§3), the method degrades gracefully when spread data is unavailable — which matters, since most archived perimeter data carries no rate-of-spread information at all. Three options, through the same code path.

### Fall back to a Euclidean metric

With no vector, set the metric to Euclidean and the method reduces exactly to the linear prototype. Endpoint-exact, monotone, continuous, and still a valid TIGN_G. This is the floor, and it is no worse than what the prototypes already produced.

### Infer the vector from the perimeter pair

The pair itself contains enough to estimate a bulk vector geometrically. Direction from the displacement between the perimeters — the bearing of maximum normal advance is more robust than the centroid shift. Magnitude from maximum normal advance divided by the interval. LB either fitted from the change in elongation or taken from wind. This makes the method applicable to any perimeter pair whatsoever.

### Take the metric from meteorology

The heading can come from modelled or observed wind at the fire rather than from the perimeters, and LB follows from wind speed through the Anderson relation directly. Magnitude can still come from the geometry. This uses data already present in any WRF-SFIRE workflow.

### The experiment this sets up

The current dataset does carry measured vectors, which makes the validation straightforward and worth doing regardless: infer the vector geometrically, compare it against the measured one, and quantify the agreement. If they match, the method extends to perimeters with no spread data — a broader contribution than the vector-driven version alone.

One honesty check on that path. A vector inferred from the same pair being interpolated cannot tell you anything the endpoints did not already imply. It is a legitimate prior extracted from the data rather than circular fitting, but it does make the held-out-perimeter test in §8 essential rather than optional — that is the only test showing the inferred metric predicts something it was not given.

# 8.  Validation

| Check | What it tells you |
| --- | --- |
| Closed-form spot checks | Along û arrival time must be exactly d/(a+c), against û exactly d/(a−c), perpendicular exactly d/[b√(1−c²/a²)]. Cheap unit tests with no tolerance to argue about. |
| Euclidean regression | Run with a Euclidean metric and compare to the linear prototype; expect agreement near 1e−2. Proves the unification claim and catches almost every plumbing bug. |
| Analytic front | Point source in a homogeneous elliptical medium, compared to the analytic ellipse over the whole front — not just the axis rates. Expect about 2% mean error. |
| Endpoint residual (C2) | Jaccard index of the s = 0 and s = 1 level sets against the observations. Should be ≥ 0.99; below that is rasterisation, not method. |
| Monotonicity (C7) | Violations counted before the accumulation safeguard. Should be zero; nonzero usually means the offset was not negated consistently between the two solves. |
| Head/back advance ratio (C6) | Per frame, mean normal advance on the downwind half of the front over the upwind half; should track HB. Use this rather than centroid drift — centroid displacement between nested fronts is near zero even when the anisotropy is working. |
| Implied vs observed ROS | Head-direction transit distance over the interval, against ¦v¦. The gap quantifies how much of the observed growth the bulk vector explains — a result worth reporting, not a failure. |
| Held-out perimeter | With three or more observations, evolve across the outer pair and score against the withheld middle one. The only test that measures the method rather than the constraint. |

## Test this before building the rest

In synthetic runs the anisotropy appeared to move the intermediate fronts only modestly — a downwind-to-upwind new-area ratio of 1.06 against 0.99 for an isotropic control. Treat that number as a property of the diagnostic, not an established result: it was a coarse centroid-relative statistic, and the head/back advance ratio in the table above is the measurement that would actually resolve the effect. The mechanism is real and correctly signed — downwind cells receive a small T_f and a large T_b, so they burn early, and upwind cells the reverse.

What is genuinely true is that with both perimeters pinned as hard constraints (C2), a single bulk vector has limited room to change what happens between them, and along a straight ray outward the metric is a constant scale factor that cancels in the ratio entirely. The metric bites where the geometry creates competing claims — which part of the perimeter reaches a cell first, and how the forward and backward fields trade off on opposite sides of the fire. So expect a redistribution of timing rather than a change of extent. Run the elliptical-versus-Euclidean comparison on a real perimeter pair early: if they are nearly indistinguishable, that is a finding about how much information a bulk vector carries; if they differ visibly, it is the figure that justifies the method.

# 9.  Limits and extensions

### The spotting trade

The gauge measures spread as the crow flies in the fire metric. It therefore crosses gaps — satisfying C5 with no special handling — but it also ignores obstacles. For spotting-dominated fires this is the better assumption; for barrier-dominated ones it is not, and the two cannot both be had from a homogeneous metric.

### Non-burnable barriers

Water, rock and wide roads are a genuine constraint rather than modelled physics, so admitting them does not violate C4. But any spatial field breaks homogeneity and removes the closed form. This is the one case that justifies a front solver — a level-set integrator with the elliptical Hamiltonian, which was verified during this work to converge under refinement (7.3% → 6.1% → 1.8% mean front error as cell size goes 4 → 2 → 1 m). Build it as an alternative backend behind the same interface, never as the default.

### The ramp between the two vectors

Each perimeter carries its own vector, so the true metric varies across the interval; the method uses their mean. Honouring both makes the wavelet W(t) + t·E₀ with W(t) = t[w_i + (w_f − w_i)t/2T], which turns the gauge from a quadratic into a quartic — solvable with three or four vectorised Newton steps from the constant-drift root. Test whether it changes the answer before building it. A large disagreement in direction between w_i and w_f is more informative than the correction itself: it means the interval contains a wind shift and probably should not be evolved as one segment.

### Solver approaches tested and rejected

Recorded so they are not re-attempted. Lagged-normal Picard iteration on scikit-fmm does not converge — iterate deltas oscillate and never settle, with front errors of 50% at mild anisotropy and 172% at LB = 6. It is dangerous rather than merely wrong, because the axis rates come out nearly perfect while the front geometry is bad everywhere in between, so it passes the obvious check. The agd package’s Rander2 model is not a drop-in — the Python API installs from pip and accepts the metric, but the solver is a compiled HFM binary that pip does not ship.

# 10.  Build order

|  | Milestone | Done when |
| --- | --- | --- |
| M1 | GridSpec, equidistant projection, sdf(). Step 1. | Both perimeters rasterise onto one grid in metres and round-trip to shapefile. |
| M2 | Gauge solver and blend, Euclidean metric. Steps 3–5. | Closed-form spot checks and the Euclidean regression both pass. |
| M3 | Elliptical metric from the ROS vectors. Step 2. | Analytic-front test within 2%; head/back ratio tracks HB. |
| M4 | The §8 comparison on a real perimeter pair. | You know whether the bulk vector changes the answer, and by how much. |
| M5 | TIGN_G export and pair chaining. Steps 6–7. | SFIRE initialises without rejecting the array. |
| M6 | Figures. | One filmstrip row — plausible intermediate fronts and an exact final match — replaces both current rows. |

# 11.  References

Verify page ranges and versions of record before submission; the Javaloyes et al. entry in particular appears under more than one journal line online.

| Supports | Reference |
| --- | --- |
| Prior art — same problem, same model | Kochanski, A.K. et al. (2023). Analysis of methods for assimilating fire perimeters into a coupled fire-atmosphere model. Frontiers in Forests and Global Change 6. doi:10.3389/ffgc.2023.1203578. The bi-harmonic spline baseline this method is measured against. |
| Alternative arrival-time product | Farguell, A., Mandel, J., Haley, J., Mallia, D.V., Kochanski, A., Hilburn, K. (2021). Machine Learning Estimation of Fire Arrival Time from Level-2 Active Fires Satellite Data. Remote Sensing 13(11), 2203. doi:10.3390/rs13112203. |
| Geometric framework (§2.4–2.8) | Javaloyes, M.Á., Pendás-Recondo, E., Sánchez, M. A General Model for Wildfire Propagation with Wind and Slope. arXiv:2110.03364; doi:10.1137/22M1477866. Finsler geometry, Huygens’ principle, arrival time as geodesic distance. |
| Randers ↔ Zermelo correspondence | Bao, D., Robles, C., Shen, Z. (2004). Zermelo navigation on Riemannian manifolds. Journal of Differential Geometry 66, 377–435. Establishes the bijection between Randers metrics and self-propulsion in a drifting medium. |
| Origin of the Randers metric | Randers, G. (1941). On an asymmetrical metric in the four-space of general relativity. Physical Review 59, 195–199. |
| Fire ellipse, LB and HB (§2.5) | Finney, M.A. (1998, rev. 2004). FARSITE: Fire Area Simulator — Model Development and Evaluation. USDA Forest Service RMRS-RP-4. Eqs. 13–14; U is midflame windspeed in m s⁻¹. |
| Empirical length-to-breadth relation | Anderson, H.E. (1983). Predicting wind-driven wildland fire size and shape. USDA Forest Service Research Paper INT-305. |
| Elliptical front equations | Richards, G.D. (1990). An elliptical growth model of forest fire fronts and its numerical solution. International Journal for Numerical Methods in Engineering 30(6), 1163–1179. |
| Level-set formulation (§2.1) | Osher, S., Sethian, J.A. (1988). Fronts propagating with curvature-dependent speed: algorithms based on Hamilton–Jacobi formulations. Journal of Computational Physics 79(1), 12–49. |
| WRF-SFIRE and the TIGN_G contract | Mandel, J., Beezley, J.D., Kochanski, A.K. (2011). Coupled atmosphere-wildland fire modeling with WRF 3.3 and SFIRE 2011. Geoscientific Model Development 4, 591–610. |
| Why fast marching fails under anisotropy (§2.8, §9) | Sethian, J.A., Vladimirsky, A. (2003). Ordered upwind methods for static Hamilton–Jacobi equations: theory and algorithms. SIAM Journal on Numerical Analysis 41(1), 325–363. |
| Randers-capable anisotropic solver (§9) | Mirebeau, J.-M. (2014). Anisotropic fast-marching on Cartesian grids using lattice basis reduction. SIAM Journal on Numerical Analysis 52(4), 1573–1599. See also Mirebeau & Portegies, Hamiltonian Fast Marching, IPOL (2019). |

Numerical results quoted throughout were produced on synthetic irregular perimeters and analytic elliptical test cases, not on the Mill Fire data. The prototype scripts were treated as a description of the intended workflow rather than as a codebase to extend.